SAHAS

AppRely & MEC (Machine Essence Corporation)

In order to effectively perform diagnosis and prognosis of industrial equipment, we need several pieces of software to be developed.

- Novelty Filter (NF)

  - The main question is to evaluate if the current data has been “seen” in the past.

  - Here we could have a multidimensional RBF.  Given a Universe of Discourse (the range of the inputs, U = {u_min, u_max}, which are vectors of dimension m, and a discretization/resolution, h, we can divide each dimension into n(i) = (u_max(i)-u_min{i))/h(i) bins, and put an RBF center on each bin.  We can assume (for now) that all dimensions are partition into the same mumber of bins, h(i) = h for all i=1,…,m.

  - Then, we initialize the Novel Operator, Nu(u) = 1.

    - If we have historic data, we could go through it and where there are data points in the neighborhood of a center (it’s bin), then we make Nu(u) = 0

- Multivariate PDF

  - As we ingest data for the NF, we could update the Nu operator and also estimate the PDF (Probability Density Function) of the signals.

  - We could at this time extend the Universe of Discourse to include the output space, y(t), a p-dimensional vector.

    - H = {<u(t),y(t)>}

    - Then, discretize each space with nCenters bins, where nCenters = product {n(i)}

  - Starting with PDF(u,y) = 0 for all bins,

    - For each new point,

      - r(u,y) = ceil((newPoint-pMin)/(pMax-pMin)*N)

- u(t) Predictor

  - Here, we assume that the input vector sequence, u(t), can be modeled by an ARMA (Auto-Regressive / Moving Average).

  - Then, determine the coefficients of the ARMA model.

    - Later we will study the need for a NARMA (Nonlinear ARMA) model.

  - Predict a nHorizon points ahead with confidence intervals

  - In most cases, machines operate at quasi-steady state or set-point, so the input doesn’t change considerably.  In other case the inputs follow a prescribed sequence that in general has cyclic/periodic behavior (daily/weekly).  In those case, we can look historically at those patterns and use them as default.  Then the ARMA model would only learn the difference from the default value.

- y(t) Predictor

  - Here, we assume that the input vector sequence, y(t), can be modeled by an ARX (Auto-Regressive with eXogenous inputs) or ARMAX (Auto-Regressive / Moving Average with eXogenous inputs), where the exogenous inputs are u(t).

  - Then, as before, determine the coefficients of the model(s) and

  - Predict a nHorizon points ahead with confidence intervals

  - Here, since we have the TF (Transfer Function) algorithm in place, we could use it to estimate the expected output, $\hat{y}(t)$

  - Then, with the predicted value of u(t), $\hat{u}(t)$, we can develop the ARMAX to predict future values of y(t).

    - In the future, we sill develop a DyNet (Dynamic Network) Neural Network model.

The attached files have some initial examples of some of the code that I found.